# METHOD OF STATEMENT

Date: 22nd June 2017

Description of work

To unload 10 cases of 1 units LT4300 CNC machine from 1 x 40ft FR and 3 x 40ft HQ container, uncrate, shift and position into customer premise at ground level

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

28/06/2017   –   10:00am

Location of Operation

#### 3 Cleantech Loop #01-01

Advanced Remanufacturing And Technology Centre

Equipment Use

1 x 70 ton mobile crane

Lorry crane

Rollers/pallet truck

Wire sling

1 x forklift

Canvas/metal sheet (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist up machine case from FR container and load onto ground

- Uncrate case

- Hoist and detach machine from base

- Swing machine to loading area

- Lower down machine slowly

- Insert rollers to machine base and secured with bolts and nuts

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175