# METHOD OF STATEMENT

Date: 24th Apr 2018

Description of Work

To hoist down 2 unit machine from 3rd floor through shutter door opening by using lorry crane and put aside

Machine Spec

1 unit Spring End Grinding

Dim: 150 x 140 x 240 cm

Wt: 5, 000 kg

1 unit Lathe

Dim: 120 x 80 x 140 cm

Wt: 1, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

24/04/2018   -   Afternoon slot

Location of Operation

#### No 33 Gul Lane

Baumann Spring Company (S) Pte Ltd

Equipment Use

1 x 50 ton lorry crane

Pallet truck / rollers

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc...) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Shift out machine from production to near shutter door opening

- Hoist down machine from 3rd floor and put aside

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175