# METHOD OF STATEMENT

Date: 12th Feb 2019

Description of work

To hoist up 1 unit machine with accessories to 3rd floor by using platform and lorry crane

Machine Spec

3 Packages of 1 unit Wirecut

Dim: 188 x 115 x 111 cm / Wt: 2, 069 kgs

Dim: 105 x 70 x 200 cm / Wt: 353 kgs

Dim: 85 x 70 x 140 kgs / Wt: 114 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date Of Operation

16/02/2019

Time Of Operation

09:45 am

Location of Operation

#### No 35 Pioneer Rd North

PCI Limited

Equipment Use

2 x lorry crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd floor shutter door opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 35 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 50 ton lorry crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert pallet truck onto machine base

- Shift out machine from platform

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175