# METHOD OF STATEMENT

Date: 01st Nov 2017

Description of work

To hoist down 1 unit machine from lorry chassis, insert rollers, shift and position into production area at ground floor

Machine Spec

1 unit INTE I-300

Dim: 490 x 280 x 272 cm

Wt: 15, 200 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

01/11/2017   –   12:00 pm

Location of Operation

#### No 37 Joo Koon Rd

Energy Pipeline Services Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x low loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated loading area for the lift

- Hoist up machine from low loader chassis

- Lower down machine to near ground level

- Swing machine to loading area

- Insert rollers to machine base

- Using customer forklift to assist in pulling/pushing machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175