# METHOD OF STATEMENT

Date: 20th Nov 2017

Description of work

1) To hoist down 1 unit machine from lorry chassis, insert rollers/pallet truck, shift and position into production area at ground floor

2) To shift out 1 used Boy 22A Injection Moulding machine from production, load onto lorry chassis and deliver to new location

Machine Spec

1 unit Boy 25E Injection Moulding Machine

Dim: 230 x 105 x 180 cm

Wt: 980 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

23/11/2017   –   09:45 pm

Location of Operation

#### No 41 Shipyard Rd

Teknor Apex Asia Pacific Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated loading area for the lift

- Uncrate machine case

- Hoist up and detach machine from base

- Lower down machine to near ground level

- Swing crane to loading area

- Insert rollers/pallet truck to machine base

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175