# METHOD OF STATEMENT

Date: 28th Aug 2020

Description of work

To unload 5 cases of 1 unit machine from 1 x 40ft FR container, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit 20.33.18 CMM machine

Case 1: 450 x 280 x 115 cm / Wt: 14, 600 kg

Case 2: 320 x 140 x 290 cm / Wt: 1, 000 kg

Case 3: 470 x 105 x 85 cm / Wt: 550 kg

Case 4: 145 x 115 x 140 cm / Wt: 300 kg

Case 5: 140 x 83 x 115 cm / Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

14/08/20-15/08/20

Time of Operation

9.30am

Location of Operation

#### 51 Corporation Rd

Avitron Pte Ltd

Equipment Use

2 x 633 lorry loader

Heavy duty rollers

1 set wire/webbing sling

Shackle

Metal sheet/canvas (for epoxy floor protection)

1 x 16 ton forklift

1 x 3 ton battery forklift

1 x Scissor lifter

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading bay

- Insert wire sling under machine case

- Hoist down machine case from FR container to the ground

- Uncrate case

- Attached required lifting gears onto crane hook and secured to rigging point of machine to be lifted

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto solid wooden block

- Transport machine to factory side roller shutter using forklift to transport

- Lift down machine from lorry chassis to near ground level

- Insert rollers to machine base

- Shift machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175