# METHOD OF STATEMENT

Date: 30th Dec 2019

Description of work

To unload 1 unit office container from 1 x 40ft FR using 2 x 10ton forklift

Spec

1 unit 32ft office container

Wt: 8, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

31/12/2019

Time of Operation

09:30 am

Location of Operation

#### 51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company (S) Pte Ltd

Equipment Use

2 x 10ton forklift

Obstacles

No obstacle on operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Using of 2 x 10ton forklift to unload office container from FR

- Fork up slowly to check on balancing

- If well balanced, reverse forklift slowly away from FR

- Lower down container slowly to near ground level

- Transport and position container at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175