# METHOD OF STATEMENT

Date: 30th Dec 2019

Description of work

To unload 1 unit office container from 1 x 40ft FR using 2 x 10ton forklift

Spec

1 unit 32ft office container

Wt: 8, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

31/12/2019

Time of Operation

09:30 am

Location of Operation

#### 51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company (S) Pte Ltd

Equipment Use

2 x 10ton forklift

Obstacles

No obstacle on operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Deploy mobile crane at designated location

- Insert wire sling under machine case

- Hoist down machine case from FR container to the ground

- Uncrate machine case

- After uncrate, hoist up machine and detach from base

- Lower down machine slowly to near ground level

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Transport machine into production area at ground floor

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175