# METHOD OF STATEMENT

Date: 22nd Jan 2019

Description of work

To hoist up 1 unit machine to 2nd floor by using platform and lorry crane

Machine Spec

1 unit Fiber Laser Cutting machine

Dim: 4700 x 2300 x 1900 mm

Wt: 3, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date Of Operation

25/01/2019

Time Of Operation

09:45 am

Location of Operation

#### No 61 Yishun Industrial Pk A #02-01

Automatic Controls & Instrumentation Pte Ltd

Equipment Use

2 x lorry crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 2nd floor shutter door opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 35 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 50 ton lorry crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert rollers/pallet truck onto machine base

- Shift out machine from platform and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175