# METHOD OF STATEMENT

Date: 26th Mar 2020

Description of work

To unload 4 cases of 1 unit machine from 1 x 20ft GP and 1 x 40ft FR container, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit Mazak T-Axis CNC Lathe / Integrex J-200

Case 1: 5.46 x 2.93 x 2.92 m / Wt: 10, 930 kg

Case 2: 1.20 x 1.04 x 1.54 m / Wt: 470 kg

Case 3: 1.70 x 1.02 x 0.94 m / Wt: 230 kg

Case 4: 3.95 x 0.88 x 1.19 m / Wt: 420 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

31/03/2020   –   09:30am

Location of Operation

#### No 7 Loyang Way

NOV Elmar Pte Ltd

Equipment Use

2 x 633 lorry loader

1 set heavy duty rollers

Pallet truck

1 set wire/webbing sling

Shackle

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the lift

- Hoist up case from FR container and load onto ground

- Uncrate case

- Attached sling to crane hook and secured onto load to be lifted

- Hoist up and detach machine from base

- Swing machine to loading area

- Lower down machine to near ground level

- Insert rollers under machine base and secured with bolts and nuts

- Shift and transport machine into inside factory

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175