# METHOD OF STATEMENT

Date: 03rd Sep 2019

Description of work

To unload 2 cases of 1 unit machine from 1 x 20ft GP and 1 x 20ft FR container, uncrate, shift and position into customer premise at ground floor

Machine Spec

2 cases of 1 unit TTC10

Case 1/2: 4000 x 2650 x 2530 mm

Wt: 9, 500 kgs

Case 2/2: 4400 x 1800 x 1700 mm

Wt: 1, 800 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

10/09/2019   –   09:30am

Location of Operation

#### No 7 Loyang Way

National Oilwell Varco

Equipment Use

2 x 633 lorry loader

1 set heavy duty rollers

1 x pallet truck

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the lift

- Hoist up machine case from FR container and load onto ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine to loading area

- Lower down machine to near ground level

- Insert rollers under machine base and secured with bolts and nuts

- Transport machine into inside factory

- Shift and position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175