# METHOD OF STATEMENT

Date: 14th June 2019

Description of work

To hoist up 1 unit machine from low-bed onto lorry chassis, transport lorry into customer factory, unload using factory overhead crane

Machine Spec

1 unit Toyoda FA 630

Dim: 651 x 320 x 353

Wt: 16, 000

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

14/06/2019   -   10:00 am

Location of Operation

#### 3 Joo Koon Circle

AME

Equipment Use

1 x 50 ton mobile crane

1 x low loader

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location

- Hoist up machine from low-bed

- Load machine onto low loader chassis

- Reverse low loader to inside factory

- Using customer overhead crane to unload machine from lorry chassis

- Shift and position machine to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175