# METHOD OF STATEMENT

Date: 06 August 2015

Description of work

To unload 1 unit machine from 1 x low-loader, uncrate, shift and position into customer premise at ground level

Machine Spec

OKK Vertical Machining Center

Dim: 3260 x 2730 x 3090 mm

Net wt: 6,460 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

To be advised

Location of Operation

#### Blk 11 Kallang Place #01-01

New Universe Mfg Pte Ltd

Equipment Use

1 x lorry crane

1 x low loader

Heavy duty rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Hoist down machine case from low loader and load onto ground

- Uncrate case

- After uncrate, hoist and detach machine from base

- Swing crane slowly to near main door

- Lower down machine

- Insert rollers to machine base and secured with bolts and nuts

- Push and position machine into the designated location inside premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175