# METHOD OF STATEMENT

Date: 07th Nov 2019

Description of work

1) To shift out and hoist down 6 unit Bar Feeder from 3rd floor by using lorry crane and load onto Malaysia lorry

2) To shift out 10 unit small machine from 3rd floor to ground floor via cargo lift and load onto Malaysia lorry

Bar Feeder Spec

420 x 55 x 130 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

13/11/2019

Time of Operation

09:45 am

Location of Operation

#### Blk 11 Kallang Place #03-02

Camtech Precision Engineering

Equipment Use

1 x 633 lorry loader – PALFINGER PK92002SHH

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the area Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Shift out bar feeder from production to near swing gate opening

- Insert one end of sling to bar feeder hoisting point and another end onto crane hook

- Hoist up 1ft above ground to check on balancing

- When well balanced, hoist down bar feeder slowly to the ground

- Load bar feeder onto Malaysia lorry

- Secure with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175