# METHOD OF STATEMENT

Date: 08th Dec 2021

Description of work

To shift out and hoist down 1 unit machine from 4th floor by using lorry crane

Machine Spec

Lathe machine

Dim: 260 x 70 x 130 cm

Wt: 1, 700 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

11-18/12/21

Time of Operation

09:45 am

Location of Operation

#### Blk 11 Kallang Place #04-12

New Universe Manufacturing Pte Ltd

Equipment Use

1 x 633 lorry loader – PALFINGER PK92002SHH

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the area Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Shift out machine from production to near swing gate opening

- Insert one end of sling onto crane hook and another end to machine hoisting point

- Hoist up 1ft above ground to check on balancing

- When well balanced, hoist down machine slowly to the ground

- Load machine onto lorry chassis

- Secure machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175