# METHOD OF STATEMENT

Date: 16th July 2018

Description of Work

To shift out and hoist down 1 unit machine and 1 unit ‘A’ frame from 2nd floor through swing gate opening by using lorry crane

Machine Spec

1 unit Lathe machine

Dim: 315 x 120 x 140 cm

Wt: 1, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

21/07/2018   -   09:45 am

Location of Operation

#### Blk 115A Commonwealth Drive #02-01/09

MJ Precision Tools Pte Ltd

Equipment Use

1 x lorry crane

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lifting (loading bay near main entrance)

- Shift out machine from customer premise to near swing gate

- Insert webbing/wire sling onto machine hoisting point

- Lorry crane to hoist down machine from 2nd floor

- Load onto lorry chassis

- Secured with strapping belt prior to moving out from premise and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175