# METHOD OF STATEMENT

Date: 20th Aug 2020

Description of Work

To shift out and hoist down 1 unit Chamber from 2nd floor, transport to another unit and hoist up to 2nd floor by using platform and lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/08/2020 (Saturday)

Time of Operation

12:00 pm

Location of Operation

#### Blk 115A Commonwealth Drive

#### Micrographics Data Pte Ltd

Hoist down from #02-10

Hoist up to #02-14

Equipment Use

1 x 633 lorry loader

1 x platform

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lifting

- Hoist up platform to 2nd floor wall opening

- Shift out chamber from customer premise into platform

- Secured chamber onto platform with strapping belt

- Hoist down platform together with chamber and load onto lorry chassis

- Transport to another unit

- Hoist up platform to 2nd floor wall opening

- Shift out chamber from platform into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175