# METHOD OF STATEMENT

Date: 17th Oct 2016

Description of Work

To hoist down 1 unit Lathe machine from 2nd floor and load onto M’sia lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

22/10/2016   -   13:00 pm

Location of Operation

#### Blk 115A Commonwealth Drive #02-03

MJ Precision Tools Pte Ltd

Equipment Use

1 x 45 ton lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc...) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Shift out machine to near gate opening

- Lorry crane to hoist down machine from 2nd floor

- Lower down machine and load onto M’sia lorry

- Secured machine with belt prior to moving out from customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175