# METHOD OF STATEMENT

Date: 23rd Aug 2021

Description of Work

To hoist up 1 unit machine to 2nd floor through swing gate opening by using lorry loader

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

04/09/21 (Saturday)

Time of Operation

09:30 am

Location of Operation

#### Blk 2 Kaki Bukit Ave 1 #02-05

Venture Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck / rollers

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Insert webbing sling to machine hoisting point

- Lorry crane to hoist up machine to 2nd floor through swing gate opening

- Lower down machine to near ground level

- Insert rollers under machine base

- Shift and position machine into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175