# METHOD OF STATEMENT

Date: 28th Feb 2023

Description of Work

To hoist up 2 unit Bar Feeder to 2nd floor through swing gate opening by using lorry loader

Bar Feeder Spec

Dim: 430 x 108 x 130 cm

Wt: 400 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07/03/23

Time of Operation

09:45 am

Location of Operation

#### Blk 20 Woodlands Link #02-24/26

Fyno Precision Pte Ltd

Equipment Use

1 x 633 lorry loader – PALFINGER PK92002SHH

Pallet truck / rollers

1 set webbing sling/shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Insert webbing sling to crane hook and secured onto Bar Feeder rigging point

- Lorry crane to hoist up Bar Feeder to 2nd floor through swing gate opening

- Lower down Bar Feeder to near ground level

- Insert rollers onto Bar Feeder stand

- Shift and position Bar Feeder into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175