# METHOD OF STATEMENT

Date: 14th Mar 2019

Description of Work

To hoist up 1 unit Bar Feeder to 5th floor through swing gate opening by using lorry crane

Bar Feeder Spec

1 unit Automatic Bar Feeder Model: XT 320S

Dim: 335 x 41 x 112 cm

Wt: 300 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

21/03/2019   -   10:30 am

Location of Operation

#### Blk 20 Woodlands Link #05-38

Yangbum Engineering Pte Ltd

Equipment Use

1 x 45 ton lorry crane

Pallet truck / rollers

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Hoist up bar feeder to 5th floor through the swing gate opening

- Lower down bar feeder to near ground level

- Insert rollers to bar feeder stand

- Shift and position bar feeder into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175