# METHOD OF STATEMENT

Date: 02nd Nov 2017

Description of Work

To hoist down 7 unit Bar Feeder from 3rd floor through swing gate opening by using lorry crane, load onto lorry chassis and deliver to scrapyard

Bar Feeder Spec

Dim: 335 x 41 x 112 cm

Wt: 300 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

08/11/2017   -   09:30 am

Location of Operation

#### Blk 20 Woodlands Link #03-36

Yangbum Engineering Pte Ltd

Equipment Use

1 x 45 ton lorry crane

1 x lorry crane

Pallet truck / rollers

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety net/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc...) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Shift out bar feeder from production to near swing gate opening

- Lorry crane to hoist down bar feeder from 3rd floor

- Load bar feeder onto lorry chassis

- Secured bar feeder with scrapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175