# METHOD OF STATEMENT

Date: 29th April 2022

Description of work

To hoist up 1 unit machine to 5th floor through swing gate opening by using platform and mobile/lorry crane

Machine Spec

1 unit PM 1350 Machine

Dim: 320 x 260 x 280 cm

Wt: 7, 400 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

14/05/22 or 28 /05/22 (Saturday)

Time of Operation

09:30 am

Location of Operation

#### Blk 3 A.M.K Industrial Pk 2A #05-08

Ace United Precision Engineering Pte Ltd

Equipment Use

1 x 70 ton mobile crane

2 x 633 lorry crane

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Hoist up platform to 3rd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform to building railing/pillar to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert rollers under machine base

- Shift out machine from platform

- Transport and position machine into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175