# METHOD OF STATEMENT

Date: 11th June 2024

Description of Work

1) To hoist up 2 unit Bar Feeder to 7th floor through swing gate opening by using platform and mobile crane

2) To unload 2 unit machine at loading bay, shift and position to 7th floor via cargo lift

Ikura Bar Feeder Spec

Dim: 400 x 65 x 140 cm each

Wt: 380 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/06/24 (Sat)

Time of Operation

09:45 am

Location of Operation

#### Blk 3 A.M.K Industrial Pk 2A #07-01

JP Precision Engineering Pte Ltd

Equipment Use

1 x 50ton mobile crane

1 x 633 lorry loader

1 x platform

Pallet truck / rollers

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting area for the lift

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Setting up mobile crane with outriggers fully extend and resting on metal plate/timber mats as base plate

- Load up bar feeder onto platform

- Secured bar feeder onto platform with strapping belt

- Mobile crane to hoist up platform together with bar feeder to 7th floor swing gate opening

- Shift out bar feeder from platform

- Transport and position into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175