# METHOD OF STATEMENT

Date: 28th July 2020

Description of Work

To hoist up 1 unit CNC machine to 2nd floor through swing gate opening by using platform and lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

01-08/08/2020

Time of Operation

09:30 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #02-01

Kei & Wah Engineering Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x 45 ton lorry crane

1 x platform

Rollers/pallet truck

1 set wire sling

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy 2 x lorry crane at designated hoisting area for the lift

- Hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to competent point and secured using 1 x 45 ton lorry crane

- Use strong belt to secure front left and right of platform onto building/pillar to prevent platform swing outward

- 1 x 50 ton lorry crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175