# METHOD OF STATEMENT

Date: 19th Oct 2021

Description of Work

To hoist up 1 unit Storage Modular to 2nd floor through swing gate opening by using platform and lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

28/10/21 - 20/11/21

Time of Operation

09:45 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #02-02

Kei & Wah Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x platform

Pallet truck

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Load modular onto platform

- Secured modular onto platform with strapping belt

- Hoist up modular together with platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Use strong belt to secure front left and right of platform onto building pillar to prevent platform swing outward

- Insert pallet truck to modular base

- Shift out modular from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175