# METHOD OF STATEMENT

Date: 31st Dec 2018

Description of Work

To hoist up 1 unit machine to 3rd floor by using lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

05/01/2018   –   09:30 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #03-01

Qualitools Precision Engineering Pte Ltd

Equipment Use

1 x 45/50 ton lorry crane

Webbing sling

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Barricade off operation area

- Lorry crane to hoist up machine to 3rd floor through swing gate opening

- Lower down machine to near ground level

- Insert pallet truck to machine base

- Shift and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175