# METHOD OF STATEMENT

Date: 21st Jan 2021

Description of Work

1) To shift out small machine and accessories from 6th floor to loading bay via cargo lift

2) To hoist down machineries from 6th floor through swing gate opening by using platform and mobile crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/01/2021   -   to shift out small machine and accessories via cargo lift

30/01/2021   -   to hoist down machineries using platform and mobile crane

Time of Operation

09:30 am

Location of Operation

#### Blk 3014A Ubi Rd 1 #06-12/13

OMGA Pte Ltd

Equipment Use

2 x 50 ton mobile crane

3-4 x lorry

1 x platform

Rollers/pallet truck

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy 2 x mobile crane at designated hoisting area

- Hoist up platform to 6th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to competent point and secured using 1 x 50 ton crane

- Use strong belt to secure front left and right of platform onto building/pillar to prevent platform swing outward

- Shift out machine from production

- Load machine onto platform

- 1 x 50 ton mobile crane to hoist down machine to the ground

- Load machine onto lorry chassis

- Secured machine with strapping belt prior to being transport to new factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175