# METHOD OF STATEMENT

Date: 06 April 2015

Description of Work

To hoist up 1 unit Bar Feeder to 5th floor

Machine Spec

1 unit Bar Feeder

Dim: 403 x 57 x 132 cm

Wt: 352 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

07/04/2015

Location of Operation

#### Blk 3023 Ubi Rd 3 Ubiplex 1

AF Precision Engineering Pte Ltd

- Equipment Use

45 ton lorry crane

Webbing sling

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc...) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Barricade operation area

- Hoist up bar feeder to 5th floor opening

- Workers to insert rollers/pallet truck to machine base

- Shift and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175