# METHOD OF STATEMENT

Date: 25th June 2018

Description of Work

To hoist down 1 unit machine from 3rd floor through swing gate opening by using lorry crane

Machine Spec

1 unit Kent KGS 84 Surface Grinder

Dim: 200 x 165 x 185 cm

Wt: 2, 800 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

30/06/2018   -   09:30 am

Location of Operation

#### Blk 4010 AMK Ave 10 #03-03

Song Tat Precision Tooling

Equipment Use

1 x 45/50 ton lorry crane

1 x lorry

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Shift out machine from customer tool room to near swing gate

- Insert webbing sling to machine hoisting point

- Lorry crane to hoist down machine from 3rd floor to ground floor

- Load machine onto lorry chassis

- Secured with strapping belt prior to moving out from premise and transport to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175