# METHOD OF STATEMENT

Date: 14th July 2020

Description of work

To hoist up 1 unit machine to 2nd floor through open space opening by using lorry crane

Machine Spec

1 unit QT 250SG

Dim: 2630 x 1680 x 1703 mm

Wt: 4, 800 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/07/20   (Saturday)

Time of Operation

09:30 am

Location of Operation

#### Blk 5 A.M.K Industrial Pk 2A #02-15

Yi Fong Precision Pte Ltd

Equipment Use

2 x 633 lorry loader

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Attached lifting gears to crane hook and secure onto hoisting point of machine to be lifted

- Hoist up machine 1ft above ground to check on balancing

- When well balance, hoist up machine to 2nd floor

- Lower down machine at 2nd floor open space opening

- Insert rollers/pallet truck under machine base

- Shift and transport machine into tenant premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175