# METHOD OF STATEMENT

Date: 24th Aug 2020

Description of work

1) To hoist down 1 unit Bar Feeder from 3rd floor by using platform and mobile/lorry crane

2) To hoist up 1 unit machine and 1 unit Bar Feeder to 3rd floor by using platform and mobile/lorry crane

Machine Spec

Hoist up

1 unit Tsugami CNC Lathe

Dim: 306 x 199 x 245 cm / Wt: 4, 184 kgs

1 unit Bar Feeder

Dim: 402 x 66 x 152 cm / Wt: 908 kg

Hoist down

1 unit Bar Feeder

Dim: 355 x 57 x 132 cm / Wt: 520 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

31/08/2020 (Monday)

Time of Operation

09:30 am

Location of Operation

#### Blk 5 A.M.K Industrial Pk 2A #03-10

Illuminate Precision Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x platform

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd floor

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform to building railing/pillar to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert rollers to machine base

- Shift out machine from platform and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175