# METHOD OF STATEMENT

Date: 19th Dec 2024

Description of Work

To unload machine and accessories from 1 x 40ft HC container, hoist up to 2nd floor through shutter door opening by using platform and lorry crane

Machine Spec

Case 1: 2150 x 1500 x 2200 mm / Wt: 300 kg

Case 2: 2680 x 1680 x 2150 mm / Wt: 1, 860 kg

Case 3: 1640 x 1530 x 2150 mm / Wt: 580 kg

Case 4: 2900 x 1380 x 2100 mm / Wt: 780 kg

Case 5: 1680 x 0890 x 1080 mm / Wt: 260 kg

Case 6: 1020 x 0820 x 1470 mm / Wt: 250 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Subject to vessel arrival:

26/12/24 – 31/12/24

Time of Operation

09:45 am

Location of Operation

#### 6 Marsiling Lane #02-00

Equipment Use

2 x 633 FALFINGER lorry loader

1 x platform

Pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- 1 x lorry crane to hoist up platform to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Lorry crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert rollers under machine bed

- Shift out machine from platform

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175