# METHOD OF STATEMENT

Date: 29th Nov 2024

Description of Work

To unload machine and accessories from 1 x 40ft HC container, hoist up to 2nd floor through shutter door opening by using lorry crane

Machine Spec

Dim: 3000 x 1350 x 2100 mm / Wt: 990 kg

Dim: 2200 x 1800 x 1200 mm / Wt: 230 kg

Dim: 2100 x 1810 x 2200 mm / Wt: 730 kg

Dim: 1080 x 1720 x 850 mm / Wt: 130 kg

Dim: 2100 x 1000 x 800 mm / Wt: 110 kg

Dim: 2300 x 1500 x 1500 mm / Wt: 260 kg

Dim: 1200 x 1200 x 1200 mm / Wt: 200 kg

Dim: 2100 x 1600 x 2000 mm / Wt: 390 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Subject to vessel arrival:

06/12/24 – 09/12/24

Time of Operation

09:45 am

Location of Operation

#### 6 Marsiling Lane #02-00

Equipment Use

1-2 x 633 FALFINGER lorry loader

1 x platform

Pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Load up machine onto platform

- Secure machine onto platform with strapping belt

- Rigger to insert webbing to crane hook

- Secured sling to rigging point of platform

- Lorry crane to hoist up platform together with machine to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Use strong belt to secure front left and right of platform to building railing/pillar to prevent platform swing outward

- Shift out machine from platform

- Position machine to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175