# METHOD OF STATEMENT

Date: 06th Oct 2016

Description of work

1) To shift out 1 unit old V32 CNC machine from ground floor, hoist up, load onto low-loader and transport to new location

2) To unload 1 unit new V32 CNC machine from 1 x 40ft HQ and 1 x 20ft FR container, uncrate, shift and position into customer premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

08-26/11/2016 – 09:30 am

Location of Operation

#### Blk 8 Lorong Bakar Batu #01-02

Chen Wee Precision Engineering Works Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x low loader

1 x platform – to laid out for machine passage way

Pallet truck/rollers

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile/lorry crane at designated location for the lift

- Lay out platform for machine pathway

- Hoist down machine from FR container

- Uncrate machine case

- Hoist and detach machine from base

- Load machine onto platform

- Insert rollers to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175