# METHOD OF STATEMENT

Date: 12th Apr 2017

Description of work

To unload 3 cases of 1 unit machine from 1 x 20ft FR and 1 x 20ft GP container, uncrate, shift and position into customer premise at ground floor

Machine Spec

Roku Roku Milling machine

Dim: 2820 x 2370 x 2870 mm

GW: 6,000 kgs

NW: 5,200 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

TBA

Location of Operation

#### Blk E3 #01-06, 2 Engineering Drive 3

NUS Fabrication Support Centre

Equipment Use

1 x 50 ton lorry crane

Rollers/pallet truck

Wire/webbing sling

1 x platform (to laid out for machine pathway)

Obstacles

No obstacle on crane parking area and route to machine positioning)

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Hoist up machine case from FR container and load onto ground

- Uncrate case

- Hoist up and detach machine from base

- Load machine onto lorry chassis

- Transport machine to loading bay

- Hoist and load machine onto platform

- Insert rollers/pallet truck to machine base

- Shift and position machine to the designated location

- Unload machine from rollers and let it sit on machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175