# METHOD OF STATEMENT

Date: 05 June 2015

Description of work

To hoist down machineries from 3rd floor by using platform and mobile crane, load onto lorry and deliver to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

From 24-26/06/2015 - 10:00 am

Location of Operation

#### From No 88 International Rd – No 8 Tuas West Ave

Hunting Energy Services Pte Ltd

Equipment Use

2 x 100 ton mobile crane

Lorry crane

1 x platform

Rollers / pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# Job Scope

- 2 x 100 ton mobile crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd floor opening

- Stabilize platform and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 100 ton mobile crane

- Use strong belt to secured front left and right of platform to prevent platform swing outward

- Shift out machine from production area into platform

- Hoist down and load machine onto lorry

- Secured machine with strapping belt prior to moving out of premise to new location

- At new location, hoist down machine from lorry chassis

- Insert rollers/pallet truck to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175