# METHOD OF STATEMENT

Date: 06th Jan 2020

Description of work

To unload 1 steel case at unloading bay, uncrate, hoist and detach machine from base, shift and position into tool room at ground floor

Machine Spec

1 unit Mitsubishi Wire-Cut EDM

Dim: 399 x 216 x 225 cm

Wt: 3, 186 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

13/01/2020   –   12:00 pm

Location of Operation

#### No 1 Yishun Ave 7

Agilent Technologies (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

4ft x 8ft metal plate (to cover up drain)

Canvas, metal sheet (to lay out for rubber tiles protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Deploy lorry loader at designated unloading area

- Hoist down case from lorry chassis

- Uncrate machine case

- Hoist up and detach machine from base

- Swing machine away from base to loading area

- Lower down machine to near ground level

- Insert pallet truck to machine base

- Shift and transport machine into tools room at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175