# METHOD OF STATEMENT

Date: 12 Jan 2016

Description of work

To unload 10 cases of 1 set HBM-130 machine from 1 x 40ft FR, 1 x 20ft OT , 3 x 40ft HQ and 1 x 20ft GP container, uncrate, shift and position into customer premise at ground level

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est: 19-21/01/2016 – 10:00 am

Location of Operation

#### No 10 Gul Circle

OMS Oilfield Service Pte Ltd

Equipment Use

70 ton mobile crane

Lorry crane

1 x low loader

Heavy duty rollers/pallet truck

Wire sling

1 x Forklift

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist up case from FR container and load onto low loader

- Low loader to transport case to back of building

- Unload case from low loader onto the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane slowly away from base to loading area

- Lower down machine

- Insert heavy duty rollers to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175