# METHOD OF STATEMENT

Date: 15 Jan 2016

Description of work

To unload, shift and position machine to location by using factory overhead crane

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est: 25-26/01/2016

Location of Operation

#### No 10 Gul Circle

OMS Oilfield Services PL

Equipment Use

25 ton factory overhead crane

Wire sling

Obstacles

No obstacle on route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to move forward into inside factory

- Reverse container and park back to back with lorry

- Jack up and level lorry chassis with container

- Lorry crane to pull out machine case from container and load chassis

- Using factory overhead crane to hoist down case to the ground

- Un-crate machine case

- Overhead crane to hoist and detach machine from base

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175