# METHOD OF STATEMENT

Date: 09th Nov 2022

Description of work

To unload 2 packages of 1 unit machine from 1 x 40ft OT and 1 x 20ft FR container, uncrate, shift and position machine to location by using factory overhead crane

Machine Spec

1 unit Everising VBS1013-60 Band Saw machine

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

12/11/22

Time of Operation

09:45 am

Location of Operation

#### 10th Sixth Lok Yang Rd

Kim Ann Engineering Pte Ltd

Equipment Use

Factory overhead crane

1 x 633 lorry loader

Wire/webbing sling

Shackle

Obstacles

No obstacle on route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of manpower to site

- Reverse container into inside factory premise

- Attached sling to overhead crane hook and secured to rigging point of machine

- Using factory overhead crane to hoist down machine from container

- Lower down machine to near ground level

- Swing crane to machine location

- Unload and position machine at the designated location

- Release all sling from machine rigging point

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175