# METHOD OF STATEMENT

Date: 03rd May 2023

Description of work

To hoist down 1 unit machine from lorry chassis, shift and position into customer premise

Machine Spec

1 unit EDM machine

Dim: 175 x 105 x 200 cm

Wt: 1, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/05/23

Time of Operation

11:30 am

Location of Operation

#### No 100 Seletar Aerospace View

Vector Aerospace Asia Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to crane hook and secured onto rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into factory premise

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175