# METHOD OF STATEMENT

Date: 25th Jan 2018

Description of work

To unload 1 set of machinery from 1 x 40ft HQ container, uncrate, shift and position into premise

Machine Spec

As per attached packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

01/02/2018    –    09:45am

Location of Operation

#### No 12 Jalan Tukang Innovation Drive

Dorma Production GmbH & Co KG

Equipment Use

1 x lorry crane

Rollers/pallet truck

Metal plate/canvas (for epoxy flooring protection)

Wire/webbing sling

1 x Forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift driver will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location (back to back with container)

- Pull out machine from container

- Forklift to fork down machine from lorry chassis

- Uncrate machine case

- Using crane or forklift to detach machine from base

- Unload machine at unloading area

- Insert rollers/pallet truck to machine base

- Transport machine into production room

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175