# METHOD OF STATEMENT

Date: 27th Feb 2024

Description of work

To unload 2 cases from lorry chassis, uncrate shift & position machine to designated location

Risk Assessment

A copy of Risk Assessment will be attached

Machine Spec

2 cases of 1unit New CMM

Crate 1: 184 x 176 x 200 cm, 2300 kg

Crate 2: 181 x 131 x 142 cm, 200 kg

Date of Operation

07/03/24

Time of Operation

Afternoon schedule

Location of Operation

#### No 12 Science Park Drive

DOS National Laboratories

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Hoist down case from lorry chassis to the ground

- Uncrate case

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist and detach machine from base

- Load onto machine stand

- Insert pallet truck onto machine stand

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175