# METHOD OF STATEMENT

Date: 16th Jan 2019

Description of work

To unload 3 cases of 1 unit machine from 1 x 40ft and 1 x 40ft FR container, uncrate, shift and position to temporary location at ground floor

Machine Spec

Case 1: 478 x 228 x 243 cm   /   N/Wt: 3, 395 kgs

Case 2: 368 x 228 x 239 cm   /   N/Wt: 1, 150 kgs

Case 3: 408 x 238 x 243 cm   /   N/Wt: 2, 795 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

16/01/2019   -   09:30 am

Location of Operation

#### No 127 Defu Lane 10

Seng Hua Hng Foodstuff Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x 4.5 ton forklift

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location

- Hoist down case from FR container to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to loading area

- Lower down machine slowly

- Insert rollers/pallet truck under machine base

- Shift and position machine to temporary designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175