# METHOD OF STATEMENT

Date: 31st Jan 2018

Description of work

1) To unload 11 cases of 2 unit Boring Milling machine from 4 x 40ft HQ and 1 x 20ft GP container, uncrate, shift and position machine to location by using factory 50 ton overhead crane

2) On another day, shift and transport 1 unit Boring Milling CBM-135QF to another factory, unload and position into factory by using 20 ton factory overhead crane

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

25/05/2017   -   10:00 am

Location of Operation

#### 14 Gul Ave & 10 Gul Ave

Equipment Use

50/20 ton factory overhead crane

1 x 50 lorry crane

1 x low loader

Wire/webbing sling

Obstacles

No obstacle on route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry crane and park back to back with container

- Flush up and level lorry chassis with container

- Lorry crane to pull out machine case from container and load chassis

- Using factory overhead crane to hoist down case to the ground

- Un-crate machine case

- Overhead crane to hoist and detach machine from base

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175