# METHOD OF STATEMENT

Date: 31st Jan 2018

Description of work

To hoist up and load 1 unit machine Base, Column and Table onto low loader, transport to another factory, unload and position into factory by using lorry crane and factory overhead crane

Machine Spec

Boring Milling Machine / CBM-135QF

Case 6: 9950 x 2270 x 2300 mm / Wt: 14, 500 kgs

Case 9: 4180 x 2270 x 2000 mm / Wt: 9, 500 kgs

Case 11: 5400 x 2270 x 1800 mm / Wt: 22, 300 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

01/02/2018   -   10:00 am

Location of Operation

#### 14 Gul Ave & 10 Gul Ave

SWTS Pte Ltd

Equipment Use

50/20 ton factory overhead crane

1 x 50 lorry crane

1 x low loader

Wire/webbing sling

Obstacles

No obstacle on route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

14 Gul Ave

- Reverse low loader into factory

- Using 50 ton overhead crane to hoist up machine and load onto low loader

- Secured machine with strapping belt

- Transport and deliver machine to another factory

10 Gul Ave

- Reverse low loader into factory

- Using 20 ton overhead crane to hoist down machine base and column from low loader

- Shift and position onto foundation

- Using 20 ton overhead crane and 50 ton lorry crane to hoist down machine table from low loader

- Shift and position to the designated location

- To assist in installation of machine parts

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175