# METHOD OF STATEMENT

Date: 07th Sep 2017

Lorry crane activity

Transport lorry crane into customer premise

Park lorry crane at designated location

Reverse container and park back to back with lorry crane

Flush up and level lorry chassis with container

Lorry crane to pull out machine from container and load onto lorry chassis

Using 50 overhead crane to hoist down machine

Load machine onto ground

Uncrate machine case

Hoist up and detach machine from base

Shift and position machine to designated location

Date / Time of Operation

11/09/2017   -   10:00 am

Location of Operation

#### No 14 Gul Ave

SWTS Pte Ltd

Equipment Use

1 x 50 ton lorry crane

50 ton overhead crane

Wire/webbing sling

Obstacles

No obstacle on route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry crane and park back to back with container

- Flush up and level lorry chassis with container

- Lorry crane to pull out machine case from container and load onto chassis

- Using factory overhead crane to hoist down case to the ground

- Un-crate machine case

- Overhead crane to hoist and detach machine from base

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175