# METHOD OF STATEMENT

Date: 07th Nov 2016

Description of work

To unload 1 unit Mazak MEGA 8800 CNC machine from 1 x 20ft FR, 6 x 40ft FR, 1 x 20ft DC, 2 x 40ft DC and 1 x 40ft HC container, uncrate, shift and position into customer premise at ground level

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est: 15/11/2016 – 09:30am

Location of Operation

#### No 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 70 ton mobile crane

50 ton lorry crane

Heavy duty rollers

Metal plate (flooring protection)

Wire/webbing sling

1 x Forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist down machine case from container and load onto ground

- Uncrate case

- Hoist and detach machine from base

- Swing machine to loading area

- Insert heavy duty rollers to machine base

- Shift and position machine to the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175