# METHOD OF STATEMENT

Date: 12th July 2016

Description of work

To unload 1 unit machine from 1 x 40ft FR container, uncrate, shift and position into customer premise at ground level

Machine Spec

1 x RB 312

Dim: 795 x 340 x 360 cm

GW: 25, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

21/07/2016 – 10:00am

Location of Operation

#### No 16 Tuas Ave 7

ST Kinetics Integrated Engineering

Equipment Use

1 x 100 ton mobile crane

Lorry crane

Heavy duty rollers

Wire sling

1 x Forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist up machine case and load onto ground

- Uncrate case

- Hoist and detach machine from base

- Swing machine to loading area

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Shift and position machine to the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175